PRACTICAL No.-2

AIM:-

Displaying Time over 4-Digit 7-Segment Display using Raspberry Pi

Introduction:-

For displaying small amount of data we prefer to use 4-digit 7-Segment display.

7-Segment and 4-Digit 7-Segment Display Module:

7 Segment Display has seven segments in it and each segment has one LED inside it to display the numbers by lighting up the corresponding segments. Like if you want the 7-segment to display the number "5" then you need to glow segment a, f, g, c, and d by making their corresponding pins high. There are two types of 7-segment displays: Common Cathode and Common Anode, here we are using Common Cathode seven segment display.

TM1637 4-DIGIT Seven Segment Display

Components Required:-

- Raspberry Pi with Power Supply

- TM1637 4-DIGIT Seven Segment Display

- Jumper wire

Wiring Up the TM1637 4-DIGIT Seven Segment Display:-

| TM1637 BOARD PIN | PHYSICAL PIN Number | GPIO PIN Number |
| --- | --- | --- |
| CLK | 38 | GPIO 20 |
| DI0 (Data IN) | 40 | GPIO 21 |
| VCC | 2 | DC POWER 5V |
| GND | 6 | Ground |

Circuit Design:-

Python Source Code:-

Note:

For Displaying the clock and IP Address on the 4-Digit 7 Segment display we will download the special python scripts with predefine functions.

Various types of operation or function available to control the LED in that scripts.

Steps to Perform 4-Digit 7 Segment Display:-

Step1:

Create Folder under /home/pi/Documents/ 7segment by using following command

mkdir 7segment

Step2:

Download the Following file by using wget command

For Clock

wget https://github.com/timwaizenegger/raspberrypi-examples/blob/master/actor-led-7segment-4numbers/clock.py

For IP Address

wget https://github.com/timwaizenegger/raspberrypi-examples/blob/master/actor-led-7segment-4numbers/displayIP.py

OR

To download this library on to your Raspberry Pi do the following steps:

1. Open the browser on your PC and download a zip from from the given link:

https://github.com/timwaizenegger/raspberrypi-examples/tree/master/actor-led-7segment-4numbers

2. Unzip the file, copy the file on to an usb drive and copy paste the same into the /home/pi/Documents/7segment folder of your RPi.

Step3:

Open the Source Code of clock.py and displayIP.py simultaneous by using Python IDLE (3.5.6) and run the code and see the output.

Source Code:

#For Display the time

from time import sleep

import tm1637

| try: |
| --- |
|  | import thread |
|  | except ImportError: |
|  | import _thread as thread |

# Initialize the clock (GND, VCC=3.3V, Example Pins are DIO-20 and CLK21)

Display = tm1637.TM1637(CLK=21, DIO=20, brightness=1.0)

| try: |
| --- |
|  | print ("Starting clock in the background (press CTRL + C to stop):") |
|  | Display.StartClock(military_time=False) |
|  | print ('Continue Python script and tweak Display!') |
|  | sleep(5) |
|  | Display.ShowDoublepoint(False) |
|  | sleep(5) |
|  | loops = 3 |
|  | while loops > 0: |
|  | for i in range(0, 10): |
|  | Display.SetBrightness(i / 10.0) |
|  | sleep(0.5) |
|  | loops -= 1 |
|  | Display.StopClock() |
|  | thread.interrupt_main() |
|  | except KeyboardInterrupt: |
|  | print ("Properly closing the clock and open GPIO pins") |
|  | Display.cleanup() |

Run the Code:-

sudo python clock.py

For Displaying IP Address

displayIP.py

import time

import tm1637

import socket

# Initialize the display (GND, VCC=3.3V, Example Pins are DIO-20 and CLK21)

Display = tm1637.TM1637(CLK=21, DIO=20, brightness=0.3)

s = socket.socket(socket.AF_INET, socket.SOCK_DGRAM)

s.connect(("gmail.com", 80))

IP = s.getsockname()[0]

s.close()

print ("IP of RasPi is")

print IP

# Separate on comma.

numbers = IP.split(".")

# Loop and print each city name.

loops = 3

while loops > 0:

for number in numbers:

Display.ShowInt(number)

time.sleep(2)

loops -= 1

Display.Clear()

time.sleep(2)

print('done.')

Display.cleanup()

Run the Scripts:

sudo python displayIP.py

On Display it will showing first 192. Then 168. Then 0. Then 122 Continuously